Subnetting
Q. 1. Consider a network address 180.17.0.0/17. We need 490 subnets. So subnet the given the
network to fulfill the requirement. Make sure the loss of host address is minimum. Find the
following information)

a) Total Number of hosts per subnet: 2^7 -2 = 128=126 
b) Wastage of usable hosts for subnetting: (2^16 - 2)-512*(2^7 -2) =  65534 – 64512=1022

c) Suppose you are the network administrator of subnet #133.Find the following
information of your network.
i) Network ID: 180.17.66.128
ii) Subnet Mask: 255.255.128.0 
iii) Broadcast Address: 180.17.66.255
iv) First usable Host Address: 180.17.66.129
v) Last usable Host Address: 180.17.128.254

| network address | 180 | 17 | 0 | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| network address | 1011 0100 | 0001 0001 | 0000 0000 | 0000 0000 |  |  |  |
| Subnet Mask | 1111 1111 | 1111 1111 | 1000 0000 | 0000 0000 |  |  |  |
| Subnet Mask | 255 | 255 | 128 | 0 |  |  |  |
| Host address | 1011 0100 | 0001 0001 | 1000 0101 | 0000 0000 |  |  |  |
| Subnet Mask | 1111 1111 | 1111 1111 | 1000 0000 | 0000 0000 |  |  |  |
| network address | 1011 0100 | 0001 0001 | 1000 0000 | 0000 0000 |  |  |  |
| network address | 180 | 17 | 128 | 0 |  |  |  |

180.17._1 _1 _1 _1   _1 _1 _1 _1  . _1 _0 _0 _0    _0 _0 _0 _0

180.17.  0100 0010 .1 000 0000    subnet 133

|  | 180.17.66.128 |
| --- | --- |

| Broadcast Address | 1011 0100 | 0001 0001 | 0100 0010 | 1 111 1111 |
| --- | --- | --- | --- | --- |
| First usable Host Address | 1011 0100 | 0001 0001 | 0100 0010 | 1 000 0001 |
| Last usable Host Address: | 1011 0100 | 0001 0001 | 0100 0010 | 1 111 1110 |

2^9 = 512

Subnet bit =9

HpSn: 2^7 -2 = 128-2 =126

Network bit: host bit -32 bit =7-32=-25 =25

Network ID = 2^25

Subnet part 9

Host part 7

Q. 2. Consider a IP address 25.133.0.0/9. You have to create 1946 subnets. Make sure the loss of subnets
address is minimum. Find the following information

d) Total Number of hosts per subnet: 2^5 – 2 = 32 -2 =30
e) Wastage of usable hosts for subnetting: (2^16 – 2) – 2048*(2^5 -2) =65534 – 61440=4094
f) Suppose you are the network administrator of subnet #329.Find the following
information of your network.
vi) Network ID: 25.128.41.32
vii) Subnet Mask: 255.128.0.0
viii) Broadcast Address: 25.133.41.63
ix) First usable Host Address: 25.133.41.33
x) Last usable Host Address: 25.133.41.62

| IP address | 25 | 133 | 0 | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IP address | 0001 1001 | 1000 0101 | 0000 0000 | 0000 0000 |  |  |  |
| Subnet Mask | 1111 1111 | 1000 0000 | 0000 0000 | 0000 0000 |  |  |  |
| Subnet Mask | 255 | 128 | 0 | 0 |  |  |  |
| network address | 0001 1001 | 1000 0000 | 0000 0000 | 0000 0000 |  |  |  |
| network address | 25 | 128 | 0 | 0 |  |  |  |
| network address |  |  |  |  |  |  |  |

25.128.1111 1111. 111 0 0000

Subnet part 11    host part 5

subnet 2^11 =2048

25.128. 0010 1001. 001     0 0000 subnet #329              v001 0100 1001

| 25.128. 0010 1001. 0010 0000 | 25.128.41.32 |
| --- | --- |

| Broadcast Address | 0001 1001 | 1000 0101 | 0010 1001 | 0011 1111 | 63 |
| --- | --- | --- | --- | --- | --- |
| First usable Host Address | 0001 1001 | 1000 0101 | 0010 1001 | 0010 0001 | 33 |
| Last usable Host Address: | 0001 1001 | 1000 0101 | 0010 1001 | 0011 1110 | 62 |

Q. 3.Consider a network address 10.5.0.0/10. We need 4000hosts. So subnet the given the
network to fulfill the requirement. Make sure the loss of host address is minimum. Find the
following information

g) Total Number of hosts per subnet: 2^12 -2 = 4096 -2 =4094             
h) Wastage of usable hosts for subnetting: (2^16 - 2) - 16*(2^12 – 2) =30
i) Suppose you are the network administrator of subnet #1000.Find the following
information of your network.
xi) Network ID: 10.5.250.0
xii) Subnet Mask: 255.192.0.0
xiii) Broadcast Address:  10.5.250.63
xiv) First usable Host Address: 10.5.250.32
xv) Last usable Host Address: 10.5.250.62

| network address | 10 | 5 | 0 | 0 |
| --- | --- | --- | --- | --- |
| network address | 0000 1010 | 0000 0101 | 0000 0000 | 0000 0000 |
| Subnet Mask | 1111 1111 | 1100 0000 | 0000 0000 | 0000 0000 |
| Subnet Mask | 255 | 192 | 0 | 0 |
| Network ID | 10 | 5 | 250 | 0 |
| Network ID | 0000 1010 | 0000 0101 | 1111 1010 | 0000 0000 |
| Broadcast | 10 | 5 | 250 | 63 |
| Broadcast | 0000 1010 | 0000 0101 | 1111 1010 | 0011 1111 |
| F h a | 10 | 5 | 250 | 0010 0000 |
| L h a | 10 | 5 | 250 | 0011 1110 |

host 2^12 = 4096 host bit 12

10.5. 1111 0000. 0000 0000 = 10.5.255.240

Subnet part 4    host part 12                  subnet = 2^4 = 16

Total Number of hosts per subnet 2^12 -2 = 4096 -2 =4094

Host loss (2^12 - 2) - 16*(2^4 – 2)

the network administrator of subnet #1000.Find the following
information of your network.
           10.5.1111 1010.00 00 0000 = 10.5.250.0

| 512 | 1 |
| --- | --- |
| 256 | 1 |
| 128 | 1 |
| 64 | 1 |
| 32 | 1 |
| 16 | 0 |
| 8 | 1 |
| 4 | 0 |
| 2 | 0 |
| 1 | 0 |